ROS Workspace

$ mkdir -p ~/catkin_ws/src

$ cd ~/catkin_ws/

$ catkin_make

Clone repository

$ cd ~/catkin_ws/src

$ git clone https://github.com/udacity/RoboND-Kinematics-Project.git

Update all required ros dependencies and permissions

$ cd ~/catkin_ws

$ rosdep install --from-paths src --ignore-src --rosdistro=kinetic -y

$ cd ~/catkin_ws/src/RoboND-Kinematics-Project/kuka_arm/scripts

$ sudo chmod +x target_spawn.py

$ sudo chmod +x IK_server.py

$ sudo chmod +x safe_spawner.sh

Build project

$ cd ~/catkin_ws

$ catkin_make

Add to .bashrc file

export GAZEBO_MODEL_PATH=~/catkin_ws/src/RoboND-Kinematics-Project/kuka_arm/models

source ~/catkin_ws/devel/setup.bash

inverse_kinematics.launch file

demo flag is set to "false” to run new IK_server.py

Run project

$ cd ~/catkin_ws/src/RoboND-Kinematics-Project/kuka_arm/scripts

$ ./safe_spawner.sh

$ cd ~/catkin_ws/src/RoboND-Kinematics-Project/kuka_arm/scripts

$ rosrun kuka_arm IK_server.py

FORWARD KINEMATIC

Experiment with the forward kinematics environment and get familiar with the robot.

$ roslaunch kuka_arm forward_kinematics.launch

ROS makes it very easy to get the transform between any two given frames with the tf_echo command.

$ rosrun tf tf_echo base_link link_6

KR210.urdf.xacro

All the values for the robot geometry are contained inside urdf file.

It contains static and dynamic friction coefficient for the links, each link origin w.r.t to its local C.S., mass, inertia and also each JOINT type, origin, parent/child LINK, axis, and physical limits.

Taking parameter values from URDF file to write down the manipulator:

DH Table

- alpha = twist angle

- a= link length

- d= link offset

- q= joint angle

Matrix for translation and orientation

Inverse position and orientation kinematics

theta1 = atan2(Wy, Wx)

theta2 = pi/2 - angle_a - atan2((Wz - 0.75), sqrt(Wx**2 + Wy**2) - 0.35)

theta3 = pi/2 - angle_b + 0.036